# Supplementary Material for:
An open-source GCR dosimetry pipeline for Mars mission risk assessment:
organ-specific REID, SEP acute hazard, and global uncertainty quantification

Aryan Hemendra Shah
University of Miami

2026-09-19

This document contains full physics/biology derivations, extended parameter tables, and extended discussion supporting the main text, which is constrained to 3500 words (Introduction through Discussion) per the Life Sciences in Space Research author guidelines. Section, table, and equation numbers here are independent of the main text and prefixed with S.

# Supplementary Methods

## Mission Trajectory Model

A Hohmann-transfer trajectory from Earth (1.0 AU) to Mars (1.524 AU) is generated by computing heliocentric distance as a function of mission elapsed time, r(t), using Keplerian orbital mechanics:

r(t)=a(1−e2)1+ecosθ(t),

where a=(r⊕+rMars)/2=1.262 AU is the semi-major axis and e=(rMars−r⊕)/(rMars+r⊕)=0.208 is the eccentricity of the transfer ellipse. Transit duration is fixed at 259 days (one half-period of the transfer orbit). The heliocentric distance enters the dose rate only through the solar modulation potential ϕ(t); direct geometric flux dilution as r−2 is not applied separately because the force-field modulation already encodes the energy dependence of solar wind effects at the relevant energies.

## Solar Modulation Potential

The time-resolved solar modulation potential ϕ(t) is obtained from the Usoskin et al. (2017) database, which provides monthly ϕ values reconstructed from ground-level neutron monitor count rates back to 1936. For the MSL transit period (2011 November – 2012 July), values cluster around ϕ≈481±50 MV. Systematic offsets between different neutron monitor stations introduce ±50–100 MV uncertainty in the absolute ϕ calibration; this is propagated via the phi_scale LHS parameter (Table 6).

## GCR Spectrum Generation

### Force-Field Modulation

Solar modulation of the GCR flux is computed using the Gleeson–Axford (1968) force-field approximation. For an ion of charge Z and mass number A, the modulated differential flux at kinetic energy per nucleon E is:

J(E,ϕ)=JLIS(E+δE)⋅E(E+2E0)(E+δE)(E+δE+2E0),

where E0=938.272 MeV is the proton rest-mass energy, δE=(Z/A) ϕ (MeV/nucleon), and JLIS is the local interstellar spectrum (below). The ratio in Eq. [eq:ff] is the Jacobian of the energy transformation, ensuring flux conservation. The approximation is accurate for rigidities above ∼​1 GV and ϕ in 200–1500 MV (Potgieter 2013); charge-sign-dependent drift (relevant at solar minimum) is not captured, but dose here is dominated by particles above a few hundred MeV/n where this is not expected to matter materially.

### Local Interstellar Spectra

Proton and helium LIS follow Vos & Potgieter (Vos and Potgieter 2015):

JLISH(E)=CH⋅(EGeV+0.67)−3.93β2,

with CH=1.9×104 cm​−2 s​−1 MeV​−1 sr​−1. Heavy ions (C, O, Si, Fe) follow the per-species HelMod model of Boschini et al. (Boschini et al. 2020):

JLIS(s)(En)=Cs⋅(EGeV/n+δs)−αsβ2.

Table 1 lists all adopted parameters. Iron’s flatter index (αFe=2.65) gives it disproportionate high-energy flux, amplified by Z2 stopping-power scaling.

LIS spectral parameters used in gcr/spectrum.py. Proton and helium from Vos & Potgieter (Vos and Potgieter 2015); heavy ions from Boschini et al. (Boschini et al. 2020).

| Species | Cs​ | αs​ | δs (GeV/n) | Primary source |
| --- | --- | --- | --- | --- |
| H | 1.90×104​ | 3.93 | 0.67 | Vos & Potgieter (2015) |
| He | 1.82×103​ | 2.78 | 0.62 | Vos & Potgieter (2015) |
| C | 2.50×102​ | 2.72 | 0.50 | Boschini et al. (2020) |
| O | 1.80×102​ | 2.69 | 0.50 | Boschini et al. (2020) |
| Si | 3.00×101​ | 2.70 | 0.50 | Boschini et al. (2020) |
| Fe | 4.30×100​ | 2.65 | 0.50 | Boschini et al. (2020) |

### Composition-Constrained Normalization

Calibration is a two-step procedure. Step 1 (composition constraint): for each species s, a correction factor rs=(Js/JH)ACE/CRIS/(Js/JH)LIS is computed so the modulated LIS reproduces the particle-flux ratio measured by ACE/CRIS (George et al. 2009) and PAMELA (Adriani et al. 2014) at En=200 MeV/n, ϕ=481 MV (Table 2). Step 2 (global dose scale): a single global scale k=0.894 is then fit so Js←k⋅rs⋅Js reproduces D=1.84 mGy day−1 at 16 g cm−2 Al, ϕ=481 MV (Zeitlin et al. 2013). This replaces a prior six-free-parameter fit (one factor per species fit independently to the single dose scalar, with no physical composition constraint) with one free parameter whose relative species mix is externally anchored.

ACE/CRIS and PAMELA reference particle-flux ratios relative to JH=1.0, En≈200 MeV/n, ϕ≈481 MV (2011–2012 MSL epoch).

| Species | Js/JH​ | Source |
| --- | --- | --- |
| He | 6.60×10−2​ | Adriani et al. (2014) |
| C | 1.80×10−3​ | George et al. (2009) |
| O | 2.20×10−3​ | George et al. (2009) |
| Si | 1.80×10−4​ | George et al. (2009) |
| Fe | 1.40×10−4​ | George et al. (2009) |

Table 3 compares the resulting composition against the NASA Space Radiation Laboratory (NSRL) GCR simulator reference field (Slaba et al. 2016). The proton fraction is within ∼​4 points of NSRL; the HZE fraction is ∼​1.7× the NSRL reference (down from ∼​4× under the prior calibration), attributed to CSDA transport not modeling nuclear fragmentation (which converts high-LET heavy ions into lower-LET secondaries in NSRL/HZETRN but not here). The helium fraction is below NSRL, reflecting the measured ACE/CRIS He/H ratio at 200 MeV/n being lower than the LIS model’s He/H at the higher energies that set the unmodulated normalization.

Per-charge-group absorbed dose fractions: composition-constrained pipeline vs. NSRL reference (Slaba et al. 2016). Geometries differ by 4 g cm−2 Al and solar phase; evaluating the pipeline at NSRL conditions (20 g cm−2 Al, ϕ≈300 MV) gives HZE=12.6% vs. NSRL’s 7.6%.

| Particle class | Pipeline (16 g cm−2 Al, ϕ=481) | NSRL (20 g cm−2 Al, solar min) |
| --- | --- | --- |
| Z=1 (protons) | 68.9% | 73.3% |
| Z=2 (helium) | 9.8% | 19.2% |
| Z>2 (HZE) | 13.0% | 7.6% |

With this composition, the pipeline gives H=6.09 mSv day−1 and Qeff=3.32 at the calibration geometry, vs. RAD’s 4.81±0.97 mSv day−1 and 2.62±0.14 (a 27% excess). This is attributed to the same CSDA fragmentation gap: without converting high-LET HZE dose into lower-LET secondaries, the dose-weighted quality factor is inflated. H and REID from this pipeline should be read as conservative upper estimates. The hze_norm_scale LHS parameter (LogNormal, σ=0.20) propagates the residual ∼​20% ACE/CRIS inter-instrument composition uncertainty (George et al. 2009; Boschini et al. 2020).

## Charged Particle Transport

### Stopping Power and CSDA Range

Energy loss is computed via CSDA. Stopping power for charge Z, velocity β:

S(E/A, Z)=zeff2(E/A, Z)⋅Sp(E/A),

with Barkas effective charge (Barkas 1963) zeff=Z[1−exp(−125βZ−2/3)] (reducing to Z2 scaling above ∼​1 GeV/n). Proton stopping powers (Al, PE, water, tissue) are tabulated from NIST PSTAR/ASTAR (500 log-spaced points, 1 MeV–105 MeV, cubic-spline interpolated). CSDA range R(E)=0EdE′/S(E′,Z); exit energy after slab x is Eout=R−1(R(Ein)−xeff) with xeff=x⋅Z2/A; particles with R(Ein)<xeff stop.

### Nuclear Interaction Attenuation

Inelastic nuclear interactions are modeled as exponential fluence attenuation, Φs(E,x)=Φs(0)(E)exp(−x/λs), with mean free path λs from the Bradt-Peters geometric cross-section (Bradt and Peters 1950), σ=πr02(Ap1/3+At1/3−δ)2 (r0=1.35 fm, δ=0.83) — the same geometric form used to build HZETRN’s tabulated mean free paths (Wilson et al. 1991). This does not model the secondary fragment spectrum; above ∼​30 g cm−2 for Fe/Si this approximation becomes increasingly crude as fragmentation produces biologically-relevant lighter secondaries (C, O). A full nuclear reaction network (HZETRN, Geant4/TOPAS) would be needed to close this gap — the primary physical limitation of this transport model.

### Secondary Neutron Production

Rather than tracking individual neutron histories, neutron dose equivalent uses a pre-computed lookup, Hn(x,ϕ,mat)=HnAl(ϕ)⋅fmat⋅g(x), with material yield factor fmat (PE ≈0.72× Al) and depth dependence g(x) interpolated from the HZETRN benchmark of Slaba et al. (Slaba et al. 2014); neutron Qn=10 (ICRP-60). This introduces ±25–30% uncertainty, propagated via the neutron_H LHS parameter.

## Organ Dose Model

Organ dose is computed by transporting shielded flux through an additional tissue layer at each organ’s ICRP Publication 89 depth (ICRP 2002) (Table 4); total areal density at organ k is xtotal(k)=xwall+dk. Absorbed dose rate:

Dk=s​∫Φs(E;xtotal(k),ϕ)⋅Ss(E)⋅ρ−1⋅C dE,

with the 4π isotropic-flux factor absorbed into C=1.602×10−10 Gy per (MeV g​−1cm​−2); energy grid is 200 log-spaced points, 10–105 MeV/n (<0.5% convergence).

Organ tissue depths and ICRP-60 tissue weighting factors. Depths from ICRP Publication 89 (ICRP 2002).

| Organ | Depth (g/cm​2) | wT (ICRP-60) | Cancer type modeled |
| --- | --- | --- | --- |
| Stomach | 8.0 | 0.12 | Gastric carcinoma |
| Colon | 9.5 | 0.12 | Colorectal carcinoma |
| Lung | 5.0 | 0.12 | Lung carcinoma |
| Leukemia | 4.0 | 0.12 | Myeloid leukemia |
| Esophagus | 4.0 | 0.05 | Esophageal carcinoma |
| Liver | 6.0 | 0.04 | Hepatocellular carcinoma |
| Bladder | 10.0 | 0.04 | Transitional cell carcinoma |
| Thyroid | 2.0 | 0.04 | Thyroid carcinoma |
| Breast | 1.5 | 0.05 | Breast adenocarcinoma |
| Ovary | 12.0 | 0.20 | Ovarian carcinoma |
| Other | 7.0 | 0.05 | Composite remainder |

Dose equivalent: Hk=s​Qs⋅Dk,s, with ICRP-60 Q(L) (ICRP 1991):

Q(L)=1L<10 keV μm−10.32 L−2.210≤L≤100 keV μm−1300/LL>100 keV μm−1

applied per species at the dose-averaged LET, Qs=Q(LD,s). This mean-LET approximation (vs. integrating Q(L) over each species’ full LET spectrum) underestimates Q for carbon by ∼​16% and overestimates it for oxygen by ∼​24% (both straddle the 10 keV μm−1 kink), with ≲3% bias for H, He, Si, Fe; the opposite-signed C/O biases partially cancel in the field-averaged Qeff=∑Hk,s/∑Dk,s, which is 2.60 at the calibration geometry (before the neutron contribution raises the total apparent Qeff to 3.32), within 0.8% of the RAD ion-only estimate — a consequence of the calibration procedure, not an independent check.

## Cancer Risk Model (REID) — Full Formulation

REID integral:

REID(ae,s)=ae+LaLTμc(a,s)⋅S(a∣ae,s)S(ae∣s) da,

ae=age at exposure, L=10 yr minimum latency, S(a|ae,s)=conditional survival. Excess mortality combines EAR and ERR:

μc(a)=λcbackground(a)⋅ERR(D,ae)+EAR(D,ae,a),

with EAR/ERR organ-specific functions of dose, age at exposure, and attained age from Tables 2–3 of Cucinotta et al. (Cucinotta et al. 2013). DDREF reduces low-dose-rate risk, Deff=D/DDREF: point estimates use a fixed DDREF of 1.75; the uncertainty ensemble instead samples DDREF ∼ Uniform[1,2]. Conditional survival and baseline cancer mortality use interpolated US sex-specific lookup tables in gcr/reid.py — a practical approximation to the NASA/Cucinotta framework, not a full reproduction of underlying epidemiologic life tables.

The REID distribution is right-skewed (REID enters an exponential, 1−exp(−μ)). The single largest tail contributor is Q_factor: its LogNormal[1.0,ln2] draws reach a sampled maximum of ∼​8.2× in the N=500 ensemble, corresponding to Qeff≈27 at the calibration point — approaching but not exceeding the ICRP-60 physical maximum of 30 (L=100 keV μm−1). Every LHS sample’s Q_factor is therefore clipped so effective Q cannot exceed 30; for the reported N=500 ensemble this clip does not bind, so reported percentiles are unchanged, but it removes any possibility of an unphysical excursion at larger N or different seeds.

## Solar Energetic Particle Acute Risk

SEP fluence spectra use the Band function (Tylka and Lee 2006):

JSEP(E)=C×Eγℓexp(−E/Eb)E≤Etrans,(γℓ−γh)Ebγℓ−γhexp(γh−γℓ)⋅EγhE>Etrans,

Etrans=(γℓ−γh)Eb. Three canonical events (Table 5) use parameters from Tylka & Dietrich (Tylka and Lee 2006); Aug 1972 predates space-borne dosimetry and is reconstructed from ground-level neutron monitor/riometry data, carrying larger uncertainty (∼±50%) than the 2003/2005 events.

Band-function spectral parameters for three canonical SEP events.

| Event | C​ | γℓ​ | γh​ | Eb (MeV) | Duration (days) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Aug 1972 | 4.0×104​ | −1.78​ | −3.98​ | 32 | 2 |  |
| Oct 2003 | 1.5×104​ | −1.40​ | −3.20​ | 50 | 3 |  |
| Jan 2005 | 3.0×102​ | −0.90​ | −2.80​ | 110 | 1 |  |

BFO absorbed dose transports event-integrated proton fluence through wall + 10 g cm−2 tissue via CSDA; only protons are tracked (heavy-ion SEP fluence is generally dose-negligible at these energies). Results compare to the NASA 30-day BFO limit of 250 mGy (Cucinotta and Durante 2010). Mission encounter probability is Poisson, PSEP(T)=1−exp(−λT/365.25), λ=0.8 events/yr at solar maximum, dropping 3–5× at solar minimum (Feynman et al. 1993).

## Uncertainty Quantification via Latin Hypercube Sampling

Nine parameters are varied via Latin Hypercube Sampling (Table 6): five physics (solar modulation, LIS normalization, nuclear cross-sections, neutron yield, HZE composition) and four biology (quality factor, DDREF, ERR scale, EAR scale).

Latin Hypercube Sampling parameters, distributions, and physical justifications. LogNormal[1.0, σ] denotes a multiplicative factor exp(N(0,σ)) with median 1.0.

| Parameter | Distribution | Justification |
| --- | --- | --- |
| phi_scale | LogNormal[1.0, 0.15] | Usoskin NM inter-station spread (∼​15%) |
| LIS_norm | LogNormal[1.0, 0.12] | Voyager/AMS-02 calibration spread (∼​12%) |
| cross_sec | LogNormal[1.0, 0.20] | Nuclear σ tables: ∼​20% |
| neutron_H | LogNormal[1.0, 0.25] | HZETRN neutron table: ∼​25% (Slaba et al. 2014) |
| hze_norm_scale | LogNormal[1.0, 0.20] | ACE/CRIS inter-instrument spread (George et al. 2009; Boschini et al. 2020) |
| Q_factor | LogNormal[1.0, ln2] | ICRP-60 Q(L) form uncertainty: factor-of-2 |
| DDREF | Uniform[1.0, 2.0] | BEIR VII / ICRP-103 range |
| ERR_scale | Normal[1.0, 0.30], clipped at 0 | Cucinotta (2013) ERR fit residuals |
| EAR_scale | Normal[1.0, 0.35], clipped at 0 | Cucinotta (2013) EAR fit residuals |

Lognormal parameters cannot drive their model input negative; ERR_scale and EAR_scale are clipped at zero (affects <0.1% of draws, no visible effect on percentiles). For each LHS sample, a full mission dose integration is performed (spectrum, transport, organ dose, quality factor, REID). Publication results use N=500 (∼​90 min, single core, unparallelized); automated tests use N=20 (UNCERTAINTY_SAMPLES env var).

Sensitivity is estimated via normalized Spearman ρ2, Si=ρi2/j​ρj2 — an approximation to first-order Sobol indices that underestimates interaction effects but is tractable at modest sample counts; a proper Sobol analysis would need ∼​104 samples (Saltelli et al. 2010).

The value-of-information sweep re-runs the N=500 ensemble once per parameter, halving that parameter’s spread (sigma, or DDREF’s range width centered on the same mean) while holding all others at baseline (narrow argument to gcr.uncertainty.lhs_samples, scripts/sensitivity_narrowing_sweep.py), recording the resulting change in p95 REID.

## Dose-Averaged LET and Proton RBE Self-Consistency Check

Dose-averaged LET per species: LD,s=∫ΦsSs2 dE/∫ΦsSs dE; field value LD=s​fsLD,s, fs=Ds/s′​Ds′.

The repository also contains a proton RBE self-consistency layer using this same LD infrastructure: Wedenberg et al. (2013) (Wedenberg et al. 2013) and McNamara et al. (2015) (McNamara et al. 2015), matched to OpenTOPAS-RBE scorer conventions and checked against OpenTOPAS-RBE reference values for a 100 MeV proton beam in water (V79, α/β=1.41 Gy) — an implementation self-consistency check, not independent experimental validation. This layer is the foundation for a planned separate proton-therapy crossover study and is not a central contribution of the present paper.

# Supplementary Discussion: Full Limitations

## Transport Model Approximations

- Nuclear fragmentation secondaries. Heavy-ion nuclear interactions are treated as simple fluence removal, not as a source of lighter-ion secondaries. This underestimates secondary proton/helium flux at depth and overestimates iron attenuation, largest above 30 g cm−2. Based on the general direction/magnitude of differences between simplified transport and full fragmentation codes (Wilson et al. 1991; Slaba et al. 2014), our order-of-magnitude estimate is that omitting secondary production overestimates Qeff at 16 g cm−2 by roughly 15–25% (an extrapolation by the authors, not a value reported by either source — a rough plausibility bound). The resulting REID overestimate is likely similar in fractional magnitude, well within the 12× p5–p95 band but not negligible relative to the median.

- Delta-ray production. Secondary electrons from ion tracks are not tracked; small correction for macroscopic dose, would matter for microscopic track-structure DNA-damage models.

- Geometry simplification. Spacecraft modeled as a uniform aluminum slab; real geometries need ray-tracing (OLTARIS, Geant4).

- Stopping power uncertainties. NIST PSTAR/ASTAR accurate to ∼​1–2% above 1 MeV/n, less certain at lower energies (shell corrections).

## Spectrum and Modulation

- Monthly-resolution modulation cannot resolve daily variability. ϕ is monthly-resolution only, so the pipeline’s prediction varies smoothly within a month. Real MSL/RAD telemetry shows sub-monthly structure (counting noise, Forbush decreases, minor SEP events) the model cannot track; daily r=−0.127 (not significant, n=218) even though the multi-month trend the model targets is directionally/quantitatively consistent. Daily-resolution claims should be read as reflecting the monthly trend only.

- ISS cross-check tests modulation shape only. The ISS orbits inside Earth’s magnetosphere, attenuating GCR flux by an amount not modeled here (no geomagnetic cutoff/trapped-radiation transport). The multi-year cross-check is restricted to trend correlation, not absolute dose, and does not validate the pipeline for near-Earth or trapped-radiation environments.

- Force-field approximation. Ignores charge-sign-dependent drift/diffusion anisotropy; during solar minimum can modify the GCR spectrum below ∼​500 MeV/n by 10–20% (Potgieter 2013) — within the propagated LIS-normalization uncertainty but not explicitly modeled.

- Missing species. Only H, He, C, O, Si, Fe are modeled. Magnesium’s contribution is estimated at <3% of total dose (scaled from Boschini 2020 Mg LIS normalization); all unmodeled species (Mg, N, Ne, S, Ca, ...) combined are estimated at <5%.

- Anomalous cosmic rays. Not modeled; can contribute below ∼​100 MeV/n during solar minimum.

## Biological and Risk Model

- NASA 2023 risk framework. This pipeline implements Cucinotta (2013); NASA’s 2023 update introduces age/sex-dependent career limits and revised tissue weighting factors not yet implemented here.

- High-LET extrapolation uncertainty. EAR/ERR were fit primarily to low-LET atomic-bomb-survivor data with high-LET scaling from rodent experiments; no direct human epidemiological data exist for the HZE component of GCR. DDREF, bridging low-dose-rate space exposure and the acute exposures in the underlying datasets, is among the most debated parameters in space radiation risk assessment (Durante and Cucinotta 2011).

- Non-cancer risks. Only cancer mortality is modeled; CNS effects, cataracts, cardiovascular risk, and heritable effects are not quantified (CNS effects have drawn attention from rodent studies showing cognitive impairment below the cancer threshold (Cucinotta et al. 2014)).

- Sex and age scope. Results shown for 35-year-old male/female; a 25-year-old female has ∼​1.5–2× higher REID than a 45-year-old male for the same exposure (more remaining life-years, high breast/ovarian tissue weighting). The pipeline supports arbitrary age/sex; broader demographic analysis is future work.

# Supplementary Tables

Normalized Spearman ρ2 sensitivity indices (N=500, 35-year-old male, 259-day transit, 16 g cm−2 Al). Values sum to 1 within each column by construction. Radiobiological parameters dominate REID variance; physics parameters dominate D and H variance.

| Parameter | REID | D​ | H​ |
| --- | --- | --- | --- |
| phi_scale | 0.080 | 0.793 | 0.080 |
| LIS_norm | 0.001 | 0.188 | 0.002 |
| cross_sec | 0.020 | <0.001 | 0.008 |
| neutron_H | 0.009 | 0.001 | 0.007 |
| hze_norm_scale | 0.008 | 0.014 | 0.009 |
| Q_factor | 0.749 | 0.001 | 0.886 |
| DDREF | 0.051 | 0.002 | 0.003 |
| ERR_scale | 0.040 | 0.001 | 0.001 |
| EAR_scale | 0.043 | <0.001 | 0.005 |

Value-of-information sweep: reduction in p95 REID from halving one parameter’s uncertainty at a time (baseline p95 =20.0%).

| Parameter | Narrowed p95 | Δ p95 | % of baseline p95 |
| --- | --- | --- | --- |
| Q_factor | 13.3% | −6.7 pts | −33.5% |
| DDREF | 19.1% | −0.9 pts | −4.5% |
| phi_scale | 19.5% | −0.5 pts | −2.7% |
| ERR_scale | 19.8% | −0.3 pts | −1.3% |
| LIS_norm | 19.8% | −0.2 pts | −1.1% |
| hze_norm_scale | 19.9% | −0.2 pts | −0.8% |
| neutron_H | 20.0% | −0.1 pts | −0.3% |
| EAR_scale | 20.0% | −0.0 pts | −0.1% |
| cross_sec | 20.0% | 0.0 pts | 0.0%​ |

Comparison of GCR dosimetry tools.  = present, △ = partial, × = absent.

| Feature | Ours | HZETRN | OLTARIS | Geant4 | TOPAS |
| --- | --- | --- | --- | --- | --- |
| Open / pip-install |  | ×​ | ×​ |  |  |
| Fragmentation | ×​ |  |  |  |  |
| Organ REID |  | △​ |  | ×​ | ×​ |
| SEP library |  | △​ |  | ×​ | ×​ |
| LHS uncertainty |  | ×​ | ×​ | ×​ | ×​ |
| Clinical RBE |  | ×​ | ×​ | △​ |  |
| CI test suite |  | ×​ | ×​ | ×​ | ×​ |

Adriani, O. et al. 2014. “Time Dependence of the He/H Ratio in Cosmic Rays During the Solar Minimum Period.” The Astrophysical Journal Letters 791: L14. https://doi.org/10.1088/2041-8205/791/1/L14.

Barkas, W. H. 1963. Nuclear Research Emulsions, Vol. I: Techniques and Theory. Academic Press.

Boschini, M. J. et al. 2020. “Deciphering the Local Interstellar Spectra of Primary Cosmic-Ray Species with HelMod.” Astrophys. J. Suppl. 250: 27.

Bradt, H. L., and B. Peters. 1950. “The Heavy Nuclei of the Primary Cosmic Radiation.” Phys. Rev. 77: 54–70.

Cucinotta, F. A., M. Alp, F. M. Sulzman, and M. Wang. 2014. “Space Radiation Risks to the Central Nervous System.” Life Sci. Space Res. 2: 54–69. https://doi.org/10.1016/j.lssr.2014.06.003.

Cucinotta, F. A., and M. Durante. 2010. Risk of Radiation Carcinogenesis. NASA Johnson Space Center.

Cucinotta, F. A., M. H. Y. Kim, and L. J. Chappell. 2013. “Space Radiation Cancer Risk Projections and Uncertainties – 2012.” NASA/TP 2013-217375.

Durante, M., and F. A. Cucinotta. 2011. “Physical Basis of Radiation Protection in Space Travel.” Rev. Mod. Phys. 83: 1245. https://doi.org/10.1103/RevModPhys.83.1245.

Feynman, J., G. Spitale, J. Wang, and S. Gabriel. 1993. “Interplanetary Proton Fluence Model: JPL 1991.” J. Geophys. Res. 98: 13281.

George, J. S. et al. 2009. “Elemental Composition and Energy Spectra of Galactic Cosmic Rays During Solar Cycle 23.” The Astrophysical Journal 698: 1666–81. https://doi.org/10.1088/0004-637X/698/2/1666.

ICRP. 1991. 1990 Recommendations of the International Commission on Radiological Protection. Publication 60. International Commission on Radiological Protection.

ICRP. 2002. Basic Anatomical and Physiological Data for Use in Radiological Protection: Reference Values. Publication 89. International Commission on Radiological Protection.

McNamara, A. L., J. Schuemann, and H. Paganetti. 2015. “A Phenomenological Relative Biological Effectiveness (RBE) Model for Proton Therapy Based on All Published in Vitro Cell Survival Data.” Phys. Med. Biol. 60: 8399–416.

Potgieter, M. S. 2013. “Solar Modulation of Cosmic Rays.” Living Rev. Sol. Phys. 10: 3. https://doi.org/10.12942/lrsp-2013-3.

Saltelli, A., P. Annoni, I. Azzini, F. Campolongo, M. Ratto, and S. Tarantola. 2010. “Variance Based Sensitivity Analysis of Model Output.” Comput. Phys. Commun. 181: 259–70.

Slaba, T. C. et al. 2014. “Efficient GCR Shielding Assessment for Long Duration Exposure.” Acta Astronaut. 104: 485–90.

Slaba, T. C., S. R. Blattnig, J. W. Norbury, A. Rusek, and C. La Tessa. 2016. “Reference Field Specification and Preliminary Beam Selection Strategy for Accelerator-Based GCR Simulation.” Life Sci. Space Res. 8: 52–67.

Tylka, A. J., and M. A. Lee. 2006. “Spectral and Compositional Characteristics of Gradual and Impulsive Solar Energetic Particle Events.” Astrophys. J. 646: 1319–34.

Vos, E. E., and M. S. Potgieter. 2015. “New Modeling of Solar Minimum Proton Modulation with a Refined Diffusion Tensor.” Astrophys. J. 815: 119.

Wedenberg, M., B. K. Lind, and B. Hardemark. 2013. “A Model for the Relative Biological Effectiveness of Protons: The Tissue Specific Parameter Alpha/Beta of Photons Is a Predictor for the Sensitivity to LET Changes.” Acta Oncol. 52: 580–88.

Wilson, J. W. et al. 1991. “HZETRN: Description and Validation of a Free-Space Ion and Nucleon Transport and Shielding Computer Program.” NASA TP 3146.

Zeitlin, C. et al. 2013. “Measurements of Energetic Particle Radiation in Transit to Mars on the Mars Science Laboratory.” Science 340: 1080–84. https://doi.org/10.1126/science.1235989.